| CONTROLLED DOCUMENT | Policy and Procedure – OPERATION OF HYDRAULIC VEHICLE HOIST and WHEELCHAIR RESTRAINTS |  |  |
| --- | --- | --- | --- |
| Document Copy No. [Click here and type the # of this controlled copy] |  | Date of Approval: [Click here and type date policy originally approved] |  |
| Revision Number and Date: [Click here and type Rev.# and Rev. date - if the Original, leave blank] |  |  |  |
| File location: [Click here and type the computer file string] |  |  | Number of Pages: 3 |

OPERATION OF HYDRAULIC VEHICLE HOIST and WHEELCHAIR RESTRAINTS

# Policy

The following Procedure for the use of vehicle hoists is required to be followed by all employees when loading and unloading wheelchair consumers in and from Organisation vehicles.   The procedures follow the Organisations general commitment to safety for both consumers and staff.

The Organisation will provide training to all staff in the use of vehicle restraints and vehicle hoist operations.

This policy and accompanying procedures are based on the Australian Standards AS 2942, AS1753 & AS1745 and supporting DSC guidelines “Reducing the Risk”.

# Procedure

DEPLOYMENT OF HOIST

- Open rear van door, ensuring that consumers are clear of door.

- Clear area for the deployment of the hydraulic hoist.

- Wheelchairs to be placed in the safest position to the vehicle.

- Wheelchair brakes on and/or power off.

- Ensure consumers wheelchair restraint belts are in use.

- Plus transport belts in use if provided i.e. yellow belt/ modular/consumers or Organisation’s. Also Thoracic Straps if required.

LOADING

- Load wheelchair consumer from the safest point near the hoist of wherever possible.

- Press deploy on hand control.

- Press down on hand control. (Use hand control on fixture in vehicle or on arm of hoist. It is not necessary to move hand control from fixture.)

- PLACE REMOVABLE STEP IN POSITION Shut side door of vehicle.

- Explain to consumer what is happening. Be reassuring.

- Load wheelchair consumer onto hoist, facing forwards (except under exceptional circumstances)

- Deploy wheelchair brakes or disengage power on electric wheelchair. Chair to be positioned correctly onto hoist. Enough clearance at the front to accommodate footplates and feet away from obstructions.

- Chair is to be forward enough  for safety flaps to fold to ensure safety of  wheelchair consumer on loading.

- DO NOT LEAVE WHEELCHAIR CONSUMER UNATTENDED.

- Press up on hand control or switch located on arm of hoist.

- STAFF ARE NOT TO RIDE UP OR DOWN ON HOIST AT THE SAME TIME AS A CONSUMER IN A WHEELCHAIR.

ENTRY INTO VEHICLE

- Release brakes of the wheelchair or re-engage power to electric wheelchair.

- Manoeuvre wheelchair consumer forwards into vehicle, or if able allow consumer to move into the position you have allocated to them in the vehicle.

- Place brakes on or disengage power again to ensure consumer safety.

- Staff member gains entry into vehicle from the side door.

BELTS

- Ensure wheelchair brake is on or power off.

- Use the small grey and red floor wheelchair safety belt to secure front of wheelchair (red button to the front of vehicle)

- STRAP AROUND FRAME of wheelchair once only if there is adequate room in the vehicle.

- Ensure wheelchair belt is safely secured to the floor of vehicle.

- After securing front grey straps pull chair back to ensure that straps are taut. Brakes on.

- Place rear floor safety wheelchair belts directly behind wheelchair being secured. STRAP AROUND FRAME of wheelchair on the crossbars or any other solid part of the wheelchair, the highest point on the upright vertical pillar. (Not on the wheels or any other detachable part of the wheelchair).

- Tension with metal levers on wheelchair floor safety belts. Secure in clasp. (See attached instructions for using 4-point Webbing Wheelchair Restraint)

- Ensure that there is no movement in the wheelchair and that the wheelchair brakes are on and power disengaged where appropriate.

- Place seatbelt around the waist of the consumer, ensuring that the belt is not twisted or restricted by any wheelchair encumbrances e.g. sidearm.

- Adjust belt according to transport requirements. e.g. Buckle can be turned face down to prevent consumer from undoing it inappropriately.

- If wheelchair fitted with module strap adaptations ensure they are used.

- ENSURE REMOVABLE STEP IS IN CORRECT POSITION AT ALL TIMES.

UNLOADING

- Unbuckle all transport straps / modular straps removing any straps which may prevent clear exit and store appropriately.

- Release brakes and engage power where appropriate.

- Staff to position themselves in front of manual wheelchair consumer and at power side of electric chair.

- Explain and reassure consumer to what is happening.

- Guide wheelchair partly onto hoist.

- Put on brakes and/or disengage power.

- If one staff, exit side door and go to back of van. Release brakes, engage power. Pull wheelchair fully onto hoist.

- Reapply brakes and/or disengage power.

- If two staff, first staff member to guide wheelchair consumer partly onto hoist, second staff to pull or guide wheelchair consumer fully onto hoist.

- Reapply brakes and/or disengage power.

- Lower hoist. Release brakes / engage power.

- Remove wheelchair consumer from hoist.

- RAISE HOIST FOR NEXT TRANSFER IMMEDIATELY.

- Staff not to stand directly behind hoist but to one side.

- When taking consumer off hoist ensure that the consumer is placed in the safest position and that the wheelchair brakes applied and/or power disengaged.

- STAFF SUPERVISION IS REQUIRED. DO NOT LEAVE WHEELCHAIR CONSUMER UNATTENDED.

- STAFF ARE NOT TO TRAVEL ON HOIST TOGETHER WITH A CONSUMER IN A WHEELCHAIR.

MALFUNCTION OF HYDRAULIC HOIST

- Check that flaps on hoist release.

- Check position of red key on floor beside hoist.

- Retrieve handle off rectangular black box.

- Place handle in to red/green circle (opening facing in towards vehicle on black rectangular box. 1/4 turn to left to deploy and hoist will lower to ground by gravity.

- 1/4 turn to right to fold up - this requires leaver again to be placed in red / green circle.

- Pump until hoist folded and stored correctly.

- Store handle in bracket on front of black box.

BELT STORAGE

- When not in use between wheelchair transfers wheelchair belts to be placed in the least restrictive manner possible, paired and fastened together and stored in storage compartment of vehicle or in centre.

N.B. POWERGLIDE / GLIDE / ROLLERGLIDE WHEELCHAIR’S CAN BE TRANSPORTED IN A RECLINED POSITION.